Q01

Ans01

Q02

Ans02

Q03

Ans03-01

Ans03-02

Original control

- First we see the original PID control to the process with no decoupling.

- Org Control to variable (y1)

- Org Control to variable (y2)

Approximate ODE process with transfer function

- We could approximate ethe ODE process to transfer function with FOPDT method.

According to the formula:

Transfer func=ΔyΔu≃Ke-θsτs+1

Where

| K | Change of CV divide by change of MV |
| --- | --- |
| τ | Time constant |
| θs | Process time delay |

- Therefore, with a little bit adjustment, we can get the transfer function which can almost describe the original ODE process.

- Transfer function: (may different from different people)

| g11 | g11=1 exp⁡(-0)25s+1 |
| --- | --- |
| g12 | g12=1 exp⁡(-0)25s+1 |
| g21 | g21=5 exp⁡(-0)7s+1 |
| g22 | g22=-2 exp⁡(-0)8s+1 |

- Now we have the transfer function for each controlled variable, and manipulated variable. Decoupling can be done by the above formula. In this case, we assume that the decoupling matrix, D, is

D=1d12d211

- Therefore, we can draw the reaction curve for each decoupling manipulated variable, u*, and determine the coefficient of PID controller.

- Use Decouple variable, u1, to control y1, h.

- Use Decouple variable, u2, to control y2, Tout.

- Finally, we can implement the control system with decouple.

- We can see the control system is obvious better than the original control system.